ES3-EC LabManual

author:	Joris Geurts / Rene Bakx
saved date:	8 September 2015
reviewer:	Corné van Dijk

Table of Contents

0)	Introduction	1

0.1	Delivery	2

0.2	Arduino	2

0.3	Centipede	2

0.4	Logic analyzer	3

0.5	Free assignments	3

1)	Assignment 1: ButtonLed	4

2)	Assignment 2: TimerCTC	6

3)	Assignment 3: Hal	8

4)	Assignment 4: Digital-Analog Conversion	10

5)	Assignment 5: LowPower	14

6)	Assignment 6: Memory	17

7)	Auxiliary assignment: Serial	19

8)	Auxiliary assignment: I2C	22

9)	Appendix: pin mapping of the Atmega328P	24

10)	Appendix: Reading the buttons of the Danger Shield	25

11)	Appendix: Conversion from Hue to RGB	26

# Introduction

This document contains the practical assignments for ES3-EC. This document will get small updates, so don't use a local copy but download the latest version from the intranet. Remarks about this document are very welcome and will be incorporated in a new version as soon as possible.

For a schedule and the exact deadlines of the assignments: see Canvas

## Delivery

It is allowed to do the assignments in groups of 2 students.

A live demo during the practical hours is appreciated by the tutor.

For each assignment you simply have to hand in all information that proves that you have completed the assignment successfully. In other words: show your intelligence.

In general this consists of the following:

- source code (.ino)

- logic analyzer screen shots

- (YouTube) video (or a reminder that you already have given a live demo)

- documentation which gives an explanation of the code, interpretation of the screen shots, explanation of the video, etc.

The tutor has no telepathic capabilities, and doesn't guess what you might have done, so failing information means no information (i.e. no screen shots means you haven’t done any timing measurements, no video means you haven't done any testing, etc.).

Delivery must be done via Canvas (fhict.instructure.com). Note the following issues:

- no zip files (nor rar, etc.)

## Arduino

This course is based on an Atmega328P mounted on an Arduino Uno.

Software for the Arduino IDE can be downloaded from: http://arduino.cc/en/main/software (Sep 2013: version 1.0.5). Further instructions for installation can be found at: http://arduino.cc/en/Guide/HomePage.

Please note: if you use another CPU, or another board (like an Atmega2560), you will need another datasheet and another pin layout.

## Centipede

More information about the Centipede shield can be found at http://docs.macetech.com/doku.php/centipede_shield (the contents can also be found on intranet). Before you can use the Centipede shield, extract the Centipede library and copy it to the Arduino/libraries directory and restart the Arduino IDE.

## Logic analyzer

Software for the Intronix LA1034 can be downloaded at: http://www.pctestinstruments.com/downloads.htm (sorry, Windows only...).

Further set-up:

- Spend 5 minutes to read the Intronix tutorial via Help -> Contents -> Tutorials -> Step by Step Example

- Attach the grey wire with the black tip to pin GND (ground) of the Arduino

- check 1:

  - connect the Intronix black wire with white tip to pin 13 of the Arduino

  - on the Arduino, run the "blink" example, which continuously toggles pin 13; modify the software such that it delays only 1 ms

  - in the Intronix program, add a Signal for Data0 (the color on the screen matches the color of the physical wire) and make sure to see the Arduino's signal on your screen

- check 2:

  - connect the Intronix red wire with white tip to pin 1 (TX) of the Arduino

  - on the Arduino, run the "AnalogReadSerial" example, which continuously sends data over the serial line (the value varies when moving the Danger Shield A0 slider (if attached))

  - in the Intronix program, add an Interpreter for RS232 and Edit it: take Data2 as Signal (provided that you attached the red wire) and set Logic Sense to Positive (see screen shot)

  - make sure that you see the same text characters in the Intronix program and on the Arduino's Serial Monitor

## Free assignments

The next chapter describes the contents of the weekly assignments rather detailed and without much room for personal inventiveness. This has the advantage that you will encounter several typical embedded issues, and that you will get experience in solving them.

However, if you prefer to define your own application (e.g. because it gives much more joy to work on them), we offer you the possibility. Be sure to discuss your proposal with your tutor, and make sure that the accompanying documentation very clearly describes how you cover the same typical issues as the original assignments.

# Assignment 1: ButtonLed

Required hardware:

- Arduino

- Danger Shield

- Logic Analyzer

Assignment (required functionality)

- Write an Arduino sketch which does the following:

  - when the user presses only button D10, led D5 turns on (and the led turns off when the button is released)

  - when the user presses only button D12, led D6 turns on (and the led turns off when the button is released)
during the press, the string "Hello World!" is sent over the serial line

  - when both buttons D10 and D12 are pressed, the buzzer makes noise (for example at 200 Hz)

  - when no button is pressed, or when button D11 is pressed, nothing happens

Validation:

- On the logic analyzer show the events on the D12 button and serial line, such that you get something like:

Implementation and tips:

- it is not allowed to use Arduino library operations like  digitalRead(), digitalWrite(), pinMode(), tone()

- instead: use the registers PINB, PORTB, DDRB, etc.

- for setting/clearing bits in those registers, use the macro _BV()and the symbolic names PINB0, PINB1, etc. 
(see <arduino-dir>/hardware/tools/avr/avr/include/avr/sfr_defs.h  and .../iom328p.h)

- do not use hardcoded values for registers and bits

- Arduino library operations Serial.begin() and Serial.print() may be used

- in setup(), initialize the serial line, and initialize the required pins for input and output

- in loop(), scan the status of the input pins, and take the corresponding actions

- it is not necessary to install interrupt handlers (that's for later assignments)

Auxiliary information:

- Atmel 328P datasheet: http://www.atmel.com/Images/doc8161.pdf, chapter 13

- Arduino Pin Mapping: http://arduino.cc/en/Hacking/Atmega168Hardware

- Danger Shield info in: ’Appendix: Reading the buttons of the Danger Shield’.

- _BV macro:  http://www.micahcarrick.com/tutorials/avr-microcontroller-tutorial/avr-c-programming.html#section_5

# Assignment 2: TimerCTC

Required hardware:

- Arduino

- Danger Shield

- Logic Analyzer

Assignment (required functionality):

- Write an Arduino sketch which does the following:

  - timer 1 runs in CTC mode

  - timer 1 has a prescaler of 64

  - slider 1 determines the value of the output compare register (a full scale slider corresponds with a full scale output compare register, etc.)

  - the Compare Match A interrupt routine shows a new hex character in the 8-segment display each time ('0', '1', '2', …, 'F', '0', … etc.)
(so when the slider is at one side, the display changes slowly (approx. 4x per second), and when the slider is at the other side, the display changes too fast to observe)

  - pressing button D11 generates an interrupt; the interrupt routine turns led D5 on and off

  - pressing other buttons and moving other sliders have no action

  - the timer 1 CTC interrupt routine must also turn on and turn off the dot of the display, always in a rate of 1x per second (regardless of the position of the slider)

Tips:

- in loop(), disable interrupts (cli()) before setting a new value of the output compare register, and enable them again afterwards (sei())

- in loop(), do not always set the new value of the output compare register, but only when the slider value differs with a certain amount compared to the previous slider value
(alternative (but it is not so nice solution): use delay() with  100 ms or similar)

- investigate Arduino sketch TimerInterrupt.ino (available on the intranet), which offers the following features:

  - the timer 1 runs with a prescaler of 256

  - an interrupt is enabled for the timer overflow

  - the interrupt service routine of the timer overflow shows each time a new picture in the 8-segment display

  - an interrupt is enabled for the A1 slider

  - the interrupt service routine of the analog A1 slider turns led D6 on or off

- you will need the following registers: PCMSK0, PCIFR, PCICR, PCMSK1, TCCR1A, TCCR1B, OCR1A, TIMSK1
Table 14.8 describes the bit settings for the various timer modes. Be careful to assign the bits in the correct registers (either TCCR1A or TCCR1B)!!!

- setting and clearing of the bits in those registers must be done with the symbolic names, not with hardcoded values

- loop() polls the value of slider 1 and sets the output compare register; all other handling must be done in the interrupt routines

- the name for the CTC interrupt routine can be found in <arduino-dir>/ hardware/tools/avr/avr/include/avr/iom328p.h, search for "Timer/Counter1 Compare Match A"

- Arduino library operations analogRead(), digitalWrite(), digitalRead(), pinMode(), Serial.begin() and Serial.print() may be used

- ensure that the '8' and the 'B' have a different appearance in the 7-segment display (and the same holds for the '0' and 'D')

Auxiliary information:

- Atmel 328P datasheet: http://www.atmel.com/Images/doc8161.pdf, chapter 11, 12 and 15

- interrupt tutorial: http://www.engblaze.com/we-interrupt-this-program-to-bring-you-a-tutorial-on-arduino-interrupts/ 
Note: this tutorial says that there are only two interrupts possible (pin 2 and 3), but in reality an interrupt can be attached to all input pins.

- timer tutorial: http://www.engblaze.com/microcontroller-tutorial-avr-and-arduino-timer-interrupts/

- working example of the assignment: http://www.youtube.com/watch?v=-KZpc1X7a_E

# Assignment 3: Hal

Required hardware:

- Arduino

- Breadboard

- Jumper wires

- Ultrasonic sensor HC-SR04

- Infrared sensor

- RGB led

In this assignment you will make a Living Colors light which can be controlled with various sensors. The following functionality is required:

- The light can be controlled by moving your hand in front of the ultrasonic sensor or the infrared sensor or by specifying the RGB values in the serial monitor input.

- Switching between the three different kinds of input can be done with a simple software setting.

- The RGB led can be replaced by the serial output, which will display the RGB values.

- Switching between the two different kinds of output can be done with a simple software setting.

Below the class diagram is depicted.

Implement the following classes:

- LivingColors
This is the main class executing the setup and loop function on the Arduino.

- iSensor
This is a generic sensor interface implemented by the HardwareControl class.

- iRgb
This is a generic RGB interface implemented by the HardwareControl class.

- HardwareControl
This is the class implementing the generic interfaces.

- UltrasonicSensor
This is the driver for the utrasonic sensor.

- InfraredSensor
This is the driver for the infrared sensor.

- SerialInput

This is the driver for the serial input.

- RgbLed
This is the driver for the RGB led.

- SerialOutput
This is the driver for the serial output.

Tips:

- The Hue property of a color can be used to specify a color on a linear scale from 0 .. 360.

When the value coming from a distance sensor is interpreted a Hue value, each distance will correspond to a certain color.

- For more info on the Hue color property, see https://en.wikipedia.org/wiki/Hue.

- Use the Arduino ‘map’ function to convert the value coming from the distance sensor to a Hue value between 0 .. 360.

- Use the function of ‘Appendix: Conversion from Hue to RGB’ to convert the Hue value to a RGB value. This RGB value can be used to write to the RGB led.

- Let the iSensor interface work with RGB values so these values can be used directly to control the RGB led.

# Assignment 4: Digital-Analog Conversion

The Atmel 328P does not have a DAC, so we build our own.

Required hardware:

- Arduino

- breadboard

- many jumper wires

- multimeter

- logic analyzer

- potentiometer

- oscilloscope (available during practical hours only)

- resistor ladder 4310R-R2R LF 103 ("caterpillar")
(or many resistors)

Assignment (required functionality):

- write an Arduino sketch that generates a sine wave; the frequency of the sine wave is determined by the setting of a potentiometer

Research: first we want to make sure that this caterpillar is indeed capable of converting an 8-bit digital value into an analog signal. This can be simulated on PC: www.falstad.com/circuit.

In the circuit simulator, you design the complete ladder network, with 16 Resistors, 8 SPDT Switches (representing the 8-bit digital output port), many Wires, a Voltage Source (2 terminal), a Ground, and a Scope Probe (for measuring the analog output). This is what the circuit could look like:

In this figure, the digital value 163 (hex 0xA3) (bits 7, 5, 1 and 0 are set high) is sent into the DAC and the resulting analog output value is 3.16V (which is nearly 163/255 = 3.196V). The real time oscilloscope picture can be enabled via a right-mouse-click on the Scope Probe and selecting View in Scope. The resistors are shown as European Resistors (Option menu)

When you have built this circuit, perform 10 measurements of various 8-bit digital values and plot them in a diagram (x axis = digital value (0..255); y axis = resulting analog output (0..5V)).

Now it's time to make a real world application. Hardware setup:

- connect the potentiometer and the caterpillar to the breadboard

- connect the potentiometer to the Arduino's GND, 5V and analog pin A0

- connect the caterpillar to GND and digital outputs D7 (caterpillar's MSB), D6, D5, D4, D3, D2 (see the datasheet of the caterpillar)
note: the Arduino's D1 and D0 are used for serial communication, so connect the two LSBs of the caterpillar to GND; effectively, we have now a 6-bit DAC

caterpillar

caterpillar

in the end, it will look more or less like:

Steps for development:

- setup():

  - generate a sine wave table with 360 values: 0..359 degrees, scaled in the range 0..255

  - start timer 1 in CTC mode, with prescaler 1

  - set pins D2..D7 as output (and perhaps D8 and D9 (see above)) (the Arduino operation pinMode() may be used)

- timer ISR:

  - on each interrupt, send the next sine wave value to the output pins
this may be done by direct writing to PORTD; it is not necessary to use digitalWrite()

- loop():

  - read the value of the potentiometer (with analogRead()) and write it into OCR1A

Tips:

- reuse code from the TimerCTC assignment

- do not implement all steps together, but do your development steps separately, and test them before combining. example: print the contents of the sine wave table via the serial line (and copy it into a spreadsheet), check with a simple LED13 that the CTC works well, test your caterpillar and PORTD by sending well known values via loop(), etc.

Validation:

- use an oscilloscope, logic analyzer and perhaps your headphones

- show that the sine table generated on the Arduino is correct: print values to the serial monitor, import it into a spreadsheet and make a chart of that data

- show that the frequency of the sine is correct: compute the expected sine frequency (with clock speed, prescaler, compare register, table size, etc.) and compare it with the measured sine frequency on the oscilloscope

Bonus:

- connect the caterpillar's LSBs to Arduino's D8 and D9, and send the LSBs of a sine wave value to those pins as well (to obtain a full 8-bit DA-Conversion)

- build the ladder network with real resistors instead of the caterpillar

- access the registers for AD-conversion directly instead of using analogRead() for reading the potentiometer 
(see chapter 13 of the ATmega328P datasheet) (this will be a lot of work...)

Auxiliary information:

- information about the sine wave: https://www.sparkfun.com/tutorials/329

- information about the resistor ladder: http://en.wikipedia.org/wiki/Resistor_ladder

- information about the used caterpillar: http://www.bourns.com/data/global/pdfs/r2r.pdf and http://www.bourns.com/data/global/pdfs/4300R.pdf 
(note: the RT in the datasheet is the GND)

- working example of the application: http://www.youtube.com/watch?v=Etn9DRJZwtw

# Assignment 5: LowPower

In this assignment you will investigate the power consumption of the ATmega328P and how you can minimize it.

Required hardware:

- Arduino

- breadboard

- 6 jumper wires

- multimeter

- logic analyzer

- chip remover, e.g. a thin screw driver

- led

- resistor (approx. 1 kΩ)

This assignment is described as a list of activities you have to do. In your document, prove that you have completed them successfully. It is not necessary that all steps are combined in one Arduino sketch; e.g.: you could separate the peripheral part from the clock prescaler part.

- load the "Blink" example in the Arduino IDE; perhaps you can increase the delays to facilitate the current measurements with the multimeter

- change delay() into a new routine my_delay() which you implement with a for-loop (this technique is called "busy waiting")
 tips:

  - use the logic analyzer to find the proper value of the loop counter to achieve identical timing

  - use the volatile keyword for the loop counter to avoid unwanted compiler optimizations

note: delay() uses timer 0, which we want to turn off for power reductions in the next steps

- carefully remove the Atmega328P from the Arduino print (always: disconnect the power supply!) and attach it on the breadboard

- attach the led and the resistor to LED13 (so we don't use the LED13 on the Arduino board any more)

Atmega 328P

Atmega 328P

with only 4 jumper wires you can make the ATmega328P operational (looking approximately to this photo: )

- measure the ATmega328P current when LED13 is on and when it is off

- modify setup()of the sketch according to the following (see also paragraphs 9.9 and 9.10):

  - turn off the ADC, see registers ADCSRA, ACSR, DIDR0, DIDR1

  - turn off all timers and other peripherals, see register PRR

- again measure the ATmega328P current consumption

- the system clock can be modified with a prescaler (see register CLKPR and read paragraph 8.11 very carefully)
modify loop() such that all system clock prescaler values are set (one after another), each time followed by a delay of +/- 2 seconds (for the current measurements)
tip 1: with the highest prescaler value, the clock runs 256x slower, so your my_delay() takes 256x as much time; so we advise you to modify my_delay() such that its delay remains the same for each prescaler value.
tip 2: for setting the prescaler bits in CLKPR, you neither need to use the logical names of the bits nor the _BV() macro; it's more convenient to write direct values into the register

- measure again the current consumption for all system clock prescaler values

- of course, all current measurements are collected in a spreadsheet table, accompanied by a nice graph

- for further reducing power, let the cpu repeatingly go into a power down mode for several seconds and let itself wake up to do some work (also see paragraph 9.5):

  - at the top of the sketch, add: 
#include <avr/sleep.h>

  - in setup(), set the watchdog in Interrupt Mode and set a 4 seconds Prescaler
(see register WDTCSR; read paragraphs 10.8 and 10.9 very carefully regarding bits WDCE and WDE)

  - add an interrupt service routine for the "Watchdog Time-out Interrupt" (but its body could be empty)

  - in loop(), let the LED13 flash several times (to simulate a workload) and then bring the cpu in power down mode; this is done by:

    - setting bits in register SMCR: set the Sleep Mode to Power-Down and set Sleep Enable

    - calling the Arduino routine sleep_cpu()
(it's allowed to use sleep_cpu() because otherwise we have to program in assembly...)

  - note: it's not allowed to use Arduino library functions like set_sleep_mode() (and the others which are mentioned on http://playground.arduino.cc/Learning/ArduinoSleepCode); of course you have to set the bits in your own code

- again measure the current consumption in the Power-Down mode

Tips:

- for developing and downloading new sketches, you could put the ATmega328P back on the Arduino print (be careful with the pins, and always disconnect the power supply)

Auxiliary information:

- power tutorial: http://gammon.com.au/power

- volatile keyword: http://stackoverflow.com/questions/5822386/the-volatile-keyword-in-c-language

# Assignment 6: Memory

In this assignment you will investigate how the ATmega328P manages the memory of your program.

Required hardware:

- Arduino

Assignment:

- Write an Arduino sketch to show the location addresses of the various memory sections (such as bss, data, heap and stack) using the global variables __data_start etc.

- Furthermore, define variables and create objects that you find appropriate and show that they are located in the correct memory sections.

- Print the addresses of various functions (this can simply be done by Serial.print()).

- Also provide a stack dump and give an interpretation of the contents (in particular: local variables and return addresses).

- For all those activities: demonstrate that the program source code and its serial monitor output match with the output of avr-readelf (and probably avr-objdump).

- Preferred delivery: one document containing snippets of program code, snippets of serial monitor output, snippets of avr-readelf (and probably avr-objdump) output, together with your interpretations (colors/circles/arrows will help for sure).

Tools:

- Arduino IDE: install version 1.5.x (in addition to 1.0.x). This version has the possibility to tweak the compilation options, in particular: turn off the code optimizations.
In file <arduino-dir>/hardware/arduino/avr/platform.txt, in compiler.cpp.flags remove the option -Os (don't forget to undo this operation for the Laundry Machine project, otherwise you might run out of program space)

- avr-readelf: gives an overview of the memory sections; example usage in a terminal window:
avr-readelf -a -W yourElfFile > elf.txt

- avr-objdump: gives the assembly listing of your c++ code; example usage:
avr-objdump -d yourElfFile > asm.txt

- both tools are located in <arduino-dir>/hardware/tools/avr/bin

Implementation and tips:

- In the Arduino IDE, turn on the verbose compilation output (via File -> Preferences) such that you can see the required compilation flags (such as: the removed -Os) and the location of the elf file.

- For all functions and variables: give recognizable names (e.g. with underscores and your initials), such that you can locate them easily in the output of the avr-readelf and avr-objdump tools.

- Of course all addresses and values are printed in hexadecimal notation.

- Regarding the stack dump:

  - use register SP

  - give a nesting of at least 4 functions

  - ensure that your functions are small in size

  - ensure that your functions all have a local parameter (initialized)

  - at the deepest level: do not print (with Serial.print()) the stack (costs too much code; you might lose the overview); instead: copy the contents of the stack into a global array and print it later

  - notice that the Atmel CPU uses 1-bit shifted code addresses, compared with the map file and assembly file

Note: it has already been said in the initial chapter of this document: only delivering the code and the elf-dump is not sufficient. It must be accompanied by a clear explanation and interpretation.

Bonus:

- How are parameters are passed in function calls (for byte, int, long int and larger types (like a struct))? And how is a return value passed to the calling function (also for various types)?
(see paragraph 6.4 of the ATmega328P datasheet for a little help)

- Where are const variables located? 
(tip: if you think that they are treated as normal variables, remove the address operator ('&') (perhaps used in your Serial.print()) and examine the assembly again)

- How are parameters passed in the assembly? (bytes, integers, pointers, objects, structs)

- Does the ATmega have a Harvard or a Von Neumann architecture?
(tip: it is regarding the way how you can address the program section and the data section)

- How can memory get fragmented?

Of course, you don't answer those questions by a copy&paste from Wikipedia. Instead: write an Arduino program in which you demonstrate your knowledge about those issues, and explain the results with the help of the assembly listing.

Auxiliary information:

- http://www.nongnu.org/avr-libc/user-manual/malloc.html (first paragraph only)

- http://www.learncpp.com/cpp-tutorial/79-the-stack-and-the-heap/

- http://jeelabs.org/2011/05/22/atmega-memory-use/

- http://www.avr-developers.com/mm/memoryusage.html

- https://github.com/queezythegreat/arduino-cmake

- http://www.avr-tutorials.com/general/welcome-general-information-page

# Auxiliary assignment: Serial

In this assignment you will realize the serial communication without the Arduino library.

Sending can be implemented in two ways:

- loop() sends one character, and polls until it is transmitted. Then it sends the next character, etc.

- via interrupts
a principle which is similar to a chain smoker: the smoker lights a new cigarette with the tip of his old cigarette. In Arduino terms: the interrupt indicating that a byte has been transmitted initiates the transmission of the new character; and when it has been transmitted, a new interrupt occurs, etc. In this way, loop() only needs to send the first byte, and it doesn't need to bother about the rest of the bytes.

Required hardware:

- Arduino

- Logic Analyzer

Assignment:

- Write an Arduino sketch with the following functionality:

  - a 1-second CTC timer which updates an internal clock (counting seconds, hours and minutes)

  - a loop() which prints a long line, containing the current time; 
the printing is initiated when a counter has reached a certain value (to avoid that the lines are overlapping each other)

  - line printing can be done in two modes:

    - polling mode where loop() sends all characters (and between the characters it keeps polling until the transmission has finished)

    - interrupt mode where loop() only sends the first character, and the interrupt handler for transmitting bytes sends the other characters of the line

  - loop() toggles a GPIO (to simulate a workload)

  - an interrupt handler for receiving serial bytes (e.g. sent by your desktop's Serial Monitor). The following characters have to be handled: 'h', 'm', 's' to increment the current hours, minutes and seconds (i.e. to adjust the clock to the current time), 'p' to set the line printing in polling mode, 'i' to set the line printing in interrupt mode.
Other characters can be ignored (unless you add some other interesting functionality)

  - the serial communications is performed at 1200 baud

Implementation and tips:

- make a copy of your directory with the Arduino environment and (in this copy) remove the Rx ISR in file .../hardware/arduino/cores/arduino/HardwareSerial.cpp (around line 104)

- use ES3-EC_SerialTemplate (available on the portal) as a starting point

- add a USART Rx Complete and a USART Tx Complete ISR in your own program

- in setup(), set all necessary bits for registers UCSR0A, UCSR0B and UCSR0C
advised settings:

  - Multi-processor Communication Mode is off

  - Character Size is 8-bit

  - Mode is Asynchronous USART

  - Parity Mode is Disabled

  - Stop Bit(s) is 1-bit

- for calculating the baud rate, use the formula from paragraph 19.3.1 (and not the table from 19.11)
use macro F_CPU for the System Oscillator clock frequency (which is 16MHz)

- of course you may study the contents of HardwareSerial.cpp, but do not use the baud rate formula from HardwareSerial::begin(), because it differs from the datasheet

- in your delivery, provide similar video's and logic-analyzer screen shots as you can see below; moreover: try to capture a screen shot in which Rx Complete and Tx Complete interrupts are both visible

Explanation of http://youtu.be/xOFFUzXpwv0:

- led13 is toggled in the CTC timer

- the 3-color led simulates the workload (you don't need to implement it in this way)

- during serial transmission in interrupt mode, the 3-color led keeps changing

- during serial transmission in polling mode, the 3-color led stalls

- characters 'h', 'm' and 's' affect the current time

Logic analyzer screen shots:

used signals:

- Data0 = RS232

- Data4 = toggle at beginning of next line printing (to facilitate triggering)

- Data2 = toggle in Tx Complete ISR

- Data3 = toggle in CTC timer ISR

- Data1 = toggle in loop() (to simulate workload) (it's not always enabled because it fills the internal memory buffer of the logic analyzer very quickly)

Data2 corresponds with sent characters

CTC timer operates at 1Hz

sending in interrupt mode: Data1 continues even during transmission

sending in polling mode: during the transmission, loop() cannot do anything else (Data1 is idle)

during ISRs, the Data1 is stalled for a short moment

zooming in on Data1 (the Sample Rate of the logic analyzer is increased to 500MHz)

# Auxiliary assignment: I2C

Write an application for i2c communication with a Centipede where the i2c communication is not done with the help of the i2c module of the processor (nor by an Arduino library like Wire.h), but instead, completely done by software handling of the GPIO-pins.

Required hardware:

- Arduino

- Centipede

- logic analyzer

- bread board

- jumper wires

- buttons, leds

Assignment:

- write an Arduino application such that it communicates the i2c-protocol via digital pin 6 and 7 to the Centipede

- this application must be able to handle both settings of the Centipede's ADDRESS jumper, so it needs to find out which i2c slave addresses do and don't respond

- the Arduino can configure some of the Centipede's pins as input, and others as output (this might be hardcoded)

- the Centipede's outputs may be simply the copy of the Centipede's input, but a more advanced application is welcome

- digitalRead() and digitalWrite() may be used

Tips:

- read the datasheet of the MCP23017

- make separate routines to send an i2c clock-bit, data-bit, to read the ack, to write a byte, to read a byte etc.

- make sure that you enable the pull up resistor when reading a bit

- it seems that the MCP23017 sometimes pulls down the i2c lines (probably caused by an incorrect implementation of your i2c protocol); in such a case: reset the device by a power cycle, and improve your code (the exact situations are not known; this is more or less warning if you encounter such a situation)

Bonus

- on the Arduino, an ISR is installed to monitor the Centipede's INTA, such that changes of inputs generate interrupts. in this way, the Arduino doesn't need to poll all inputs, but it only needs to react on the interrupt

- don't use the digitalRead() and digitalWrite(), but instead use the register PINx, PORTx, DDRx; and compare the data speeds in both solutions

# Appendix: pin mapping of the Atmega328P

# Appendix: Reading the buttons of the Danger Shield

There are two versions of the Danger Shield:
One version with a knock sensor in which the buttons already have a pullup resistor and reading a button will give a ‘HIGH’ when the burtton is pressed.

The other version with a cap sensor does not have pullup resistors so you have to enable these with an extra instruction on the Arduino. Reading a button will give a ‘LOW when the button is pressed.

| Version Danger shield | Code for reading a button. |
| --- | --- |
| Knock sensor | const int buttonPin = 2; int buttonState; int counter = 0; void setup() { pinMode(buttonPin, INPUT); } void loop(){ buttonState = digitalRead(buttonPin); if (buttonState == HIGH) { delay(180); // simple debounce counter++; // some action } } |
| Cap sensor, geen pull-up weestanden. | const int buttonPin = 2; int buttonState; int counter = 0; void setup() { pinMode(buttonPin, INPUT); digitalWrite(buttonPin, HIGH); } void loop(){ buttonState = digitalRead(buttonPin); if (buttonState == LOW) { delay(180); // Simple debounce counter++; // Some action } } |

# Appendix: Conversion from Hue to RGB

The function below can be used to convert a number between 0 .. 360 to an RGB value.
Example of how to use this function:

int rgb[3]; 
hsi2rgb(hueValue, 1, 1, rgb);

When called the function will convert the hueValue between 0 .. 360 to the corresponding RGB values. After the call the array ‘rgb’ will contain the respective values for red, green and blue, each between 0 .. 255.

Function definition of hsi2rgb():

void hsi2rgb(float H, float S, float I, int* rgb) {
  int r, g, b;

  H = fmod(H,360); // cycle H around to 0-360 degrees
  H = 3.14159*H/(float)180; // Convert to radians.
  S = S>0?(S<1?S:1):0; // clamp S and I to interval [0,1]
  I = I>0?(I<1?I:1):0;

  if(H < 2.09439)
  {
    r = 255*I/3*(1+S*cos(H)/cos(1.047196667-H));
    g = 255*I/3*(1+S*(1-cos(H)/cos(1.047196667-H)));
    b = 255*I/3*(1-S);
  }
  else if(H < 4.188787)
  {
    H = H - 2.09439;
    g = 255*I/3*(1+S*cos(H)/cos(1.047196667-H));
    b = 255*I/3*(1+S*(1-cos(H)/cos(1.047196667-H)));
    r = 255*I/3*(1-S);
  }
  else {
    H = H - 4.188787;
    b = 255*I/3*(1+S*cos(H)/cos(1.047196667-H));
    r = 255*I/3*(1+S*(1-cos(H)/cos(1.047196667-H)));
    g = 255*I/3*(1-S);
  }
  rgb[0]=r;
  rgb[1]=g;
  rgb[2]=b;
}